# GPIO User Guide

The board contains multiple programmable GPIO, each pin can be independently program. At the same time, it supports GPIO Base address mode to fast read and write.

# 1 Register

## 1.1 Global Configuration Registers（LDN=ALL）

1.1.1 Logical Device Number Register(LDN, Index=0x07)

| Bit | Description |
| --- | --- |
| 7-0 | This register is, read/write, which is to select the current logical devices. |

## 1.2 GPIO Configuration Registers (LDN=0x07)

1.2.1 GPIO Base Address MSB Register(Index=0x62)

| Bit | Description |
| --- | --- |
| 7-2 | Read only as “0h” for Base Address [15:12] |
| 1-0 | Read/write, mapped as Base Address [11:8] |

1.2.2 GPIO Base Address LSB Register(Index=0x63)

| Bit | Description |
| --- | --- |
| 7-0 | Read/write, mapped as Base Address[7:0] |

1.2.3 GPIO Group 1,2,3,4,5,6,7,8,9 Output/Input Select Regisers(Index=0xC8,0xC9,0xCA,0xCB,0xCC,0xCD,0xCE,0xCF,0xD2)

| Bit | Description |
| --- | --- |
| 7-0 | 0: Input mode 1: Output mode |

# 1.3 GPIO Level Regisers (Baseaddress + Group Offset)

| Bit | Description |
| --- | --- |
| 7-0 | 1: Higl level 0: Low level |

# 2 Program Flow

- Write enter key, enter pnp mode.

- Select GPIO device.

- Configure Input/output .

- Get Base Address.

- Exit pnp mode.

- Read/write GPIO value by access base address + group offset.

# 3 Demo(linux)